Jigging Machines

We have been supplying Auto Squid Jigging Machine with two reels which is made in Korea or made in Japan

1. Model : NBJ-2002 ( Korean-made )

2. Model : HAMADE MY-3DP ( Japan-made )

3. Model : HAMADE MY-10 ( Japan-made )

4. Model : HAMADE MY-7 ( Japan-made, new )

5. Model : SANMEI SE-UT01 ( Japan-made )

6. Other brands are available, if requested

Please your  inquiry  to Vectra company

Mrs. Schein : v-trading@farlep.net

NBJ-2002 ( Korean-made )

Model : NBJ-2002 ( Korean-made : above photo )

Characteristics
A simple operation by one touch switch
A optimum control for the field condition(40steps) 
A useful sign board for checking the water depth and successive Jerk
The best power from the AC geared motor ( jigging machine only ) 

Specification
Source : AC 190V - 260V , 60Hz
Successive Jerk : 1 or 2 / one rotation
Down Velocity : 1rpm - 110rpm
Up Velocity : 1rpm - 110rpm
Water Depth : 1meter - 999meter
Upward Force : -40steps(110kg MAX)
Dimention : 481W x 335D x 455H(mm)

Characteristics of bridge computer(bridge panel, control panel)
A centralized control of the whole jigging machine at one time
A functional operation of max 48 circuit machine by one bridge panel
Power source meters for easy operation
A indication of instant data from actual field operation
Dimention : 560W x 440D x 102H (16 circuits)
Dimention : 760W x 440D x 140H (32 circuits)
Dimention : 872W x 440D x 140H (48 circuits)

HAMADE MY-3DP

Specifications

Depth of water : 0(.0) ~ 999(.9)m
Drum revolution : 20 ~ 125rpm(both raising and lowering)
Source : 3 phase AC 220V 60Hz
Motor : Specialized geared AC Moter
Reduction Ratio : 1 : 20(1/20)
Winding power : range 0 ~ 24(approx.70kgs max.)
Jigging depth : 0(.0) ~ 999(.9)m
Dimension : 485W X 285D X 445H(mm)
Anchoring leg : 520 x 260(mm)
Weight : 75kg (without winding drum)

Centralized controller

Source : sphase AC 220V 60Hz
Operation mode : independent, concurrent, sequential, group sequence, synchro, zigzag
Grouping : 4blocks(A ~ D)
Dimention : 
420W X 420D X 110H(mm)(16cirs) 
420W X 510D X 110H(mm)(32cirs) 
Anchoring leg : 
340 x 340(16cirs)
340 x 420(32cirs)
Weight : 10kg(16cirs) 13kg(32cirs)
countof circuits : 16circuits, 32circuits

HAMADE MY-10

HAMADE MY-10 ( Made in Japan )

HAMADE MY-7

This new model has 'standard remote control unit' and 'up-and-down stop remote control unit' 

Specifications

Depth of water : 0(.0) ~ 999(.9)m
Drum revolution : 5 ~ 150 rpm( when raising 5-125rpm )
Source : 3 phase AC 220V 60Hz
Motor : Specialized geared AC Moter
Reduction Ratio : 1 : 20(1/20)
Winding power : range 0 ~ 43(approx.90kgs max.)
Jigging depth : 0(.0) ~ 999(.9)m
Dimension : 495W X 270D X 440H(mm)
Anchoring leg : 520 x 260(mm)
Weight : 69kg (without winding drum)

Centralized controller

Source : 3 phase AC 220V 60Hz
Operation mode : independent, concurrent, sequential, , synchro, zigzag
Grouping : right side and left side
Dimention : 
408W X 394D X 107H(mm)
Anchoring leg : 
340 x 300(16cirs)
Weight : 12kg
countof circuits : 16circuits

SANMEI SE-UT01

SE-UT01

Dimention : W504 x H417 x D293
Source : AC220V 60Hz
Down & up velocity : 5 - 120 rpm

SE-UT02

Dimention : W504 x H483 x D335
Source : AC220V 60Hz
Down & up velocity : 5-120 rpm