Radio Buoy & Strobe Light & Sea Light

- Radio Buoy & Sel-cal Buoy
- Strobe light
- Sea light
- Light buoy
- Fish luring light
- Other

Please your  inquiry  to Vectra company

Mrs. Schein : v-trading@farlep.net

RADIO BUOY & SEL-CALL BUOY

The Radio Buoy is a device that was developed to imprive the efficiency of the fishing industry and to prevent the washing away of fishing net. An electrical wave emitted by the Radio Buoy fasteded to the fishing net is received by the direction finder of the ship, which indicates the direction and FISH quantity, information of the Radio Buoy. 

The SEL-CALL BUOY consists of a Radio Buoy with a receiver intalled in it, and it answers only when it is called by its own ship. Thr this reason, the battery power consumed is small, and when the operating area is entered by other ships, the theft of nets and fishing gear is prevented. 
This is used together with the SEL-CALL SIGNAL GENERATOR

And Batteries are also available.

| Item | RADIO BUOY(PR-30) | SEL-CALL BUOY(PRSC-30) |
| --- | --- | --- |
| Transmitter Frequency | 1606.5kHz-2850kHz(One Frequency) | 1606.5kHz-2850kHz(One Frequency) |
| Type of emission | A1A | A1A |
| Antenna Power | 3W+20% | 3W+20% |
| Code | As Stipulated(Max.4 letters) | As Stipulated(Max.4 letters) |
| Service Range | 50-150km | 50-150km |
| Frequency Deviation | Within 50 x 1000000 | Within 50 x 1000000 |
| Band Width All otment | Within 0.5kHz | Within 0.5kHz |
| Riming of Transmitted code | (Standar)-ON SW Repetition of 1min.Transmissionandand 3min.Rest.(As Specified)-TEST SW Repetition of 1min 10min. Rest |  |
| Receiving Sensitivity |  | 10?/1M, SN 3dB |
| Receiving Frequency |  | 2331.5kHz or 1902.5kHz |
| Battery Life | 7300Hr.(Package Battery) | 7300Hr.(Package Battery) |
| Pressure Durability | 20atm.Pressures | 20atm.Pressures |
| Gross weight | 22kgs | 22kgs |
| Material of drum shell | t Stainless Steel(SUS304) | Stainless Steel(SUS304) |

LIGHT BUOY

Photo sensitive strobe marker buoy light.

- Multi-purpose buoy marker light(geo-seismic research, fishing,etc):also used to designate dangerous areas, construction and harbor work areas, roadside emergencies or can be used as a general distress marker. 
- High visibility light can be seen up to 3.7miles(6.0km)
- Lightweight, easy to install, simple to operate 
- Photocell activated: 'ON' at dusk, 'OFF' at dawn
- Material : Strong, impact resistant ABS resin and stainless steel
- Color : International Orange, Yellow, Red 
- Operation ; photocell light activated "ON-OFF" control
- Beam Angle : Omni-directional 360' 
- Lamp : Xenon flash tube

All Items have LED Lamp & Xenon Tube for Flash, and it flashes successively

1. FL-1

length : 550mm
weight : 260g

2. FL-2

Use with FS-1, or FS-2
length : 650mm
weight : 4.0kgs
visibility : 6km
battery : two "D"cell alkaline (1.5V x 2pcs or 1pcs)
battery life : about 23days
buoy size : 410mm(H) x 360mm
flash rate : 1.5sec
waterproof : 30meter

3. FL-3

Use with FS-4, or FS-7
length : 650mm
weight : 4.1kgs
visibility : 6km
battery : two "D"cell alkaline (1.5V x 2pcs)
battery life : about 20days
buoy size : 410mm(H) x 360mm
flash rate : 2.5sec
waterproof : 30meter

4.FL-4

length : 820mm
weight : 6.4kgs
visibility : 6km
battery : 6V package alkaline
battery life : about 90days
buoy size : 250mm(H) x 280mm
flash rate : 2.5sec
waterproof : 30meter

5.FL-5

LED lamps
length : 1420mm
weight : 7.5kgs
visibility : 6km
battery : 6V package alkaline
battery life : about 90days
buoy size : 250mm(H) x 280mm
flash rate : 2.5sec
waterproof : 30meter

6. FL-6

length : 1400mm
weight : 6.7kgs
visibility : 6km
battery : four "D"cell alkaline (1.5V x 4pcs)
battery life : about 40days
buoy size : 210mm(H) x 250mm
flash rate : 2.5sec
waterproof : 30meter

7. FL-7 ( Solar-cell light buoy )

made in stainless steel
length : 1800mm
weight : 30kg
lens : polycarbonate resin
float : 420mm(H) x 720mm(ABS resin)
lamp : high brightness LED, 26EA
solar-cell : 100mm x 100mm
power : 2V/5Ah rechargeable battery
battery life : over 3 years
flash rate : 0.5seconds lighting, 3.5seconds light turn off
visibility : about 2-3km
operation : on/off switich

STROBE LIGHT

Fishing Net Indicating Lights
Photo sensitive strobe marker light

- Light sensitive switch : automatically activates as nighttime approach
- Multi-purpose buoy marker light(geo-seismic research, fishing,etc):also used to designate dangerous areas, construction and habor work areas, roadside emergencies or can be used as a general distress marker. 
- Lightweight, easy to install, simple to operate 
- Photocell activated: 'ON' at dusk, 'OFF' at dawn
- Material : Strong, impact resistant ABS resin 
- Color : International Orange, Yellow, Red 
- Operation : photocell light activated "on-off" control
- Beam Angle : Omni-directional 360' 
- Visibility : Hight intensity strobe is visible up to 3.7 miles(6km) 
- Lamp : Xenon flash tube

All Items have LED Lamp & Xenon Tube for Flash

1. FS-1

LED lamps of red color + white color
Size : 280mm x 53mm
Weight : 250g
Visibility : 6km
Battery Alkaline "D"cell : two
battery life : 23days
Flash rate : 1.5sec
Waterproof : 30meter
FS-1 with mercury switch (if bottom up, the light is turned up automatically) is available too.

2. FS-2

LED lamps
Size : 204mm x 53mm
Weight : 220g
Visibility : 6km
Battery Alkaline "D"cell : one
battery life : 20days
Flash rate : 1.5sec
Waterproof : 30meter

3. FS-3

LED lamps
With FS-2
Size : 230
Weight : 340g
Visibility : 6km
Battery Alkaline "D"cell : one
battery life : 20days
Flash rate : 1.5sec
Waterproof : 30meter

With FS-1 available

4. FS-4

LED lamps of red + white
Size : 260mm x 56mm
Weight : 280g
Visibility : 6km
Battery Alkaline "D"cell : two
battery life : 30days
Flash rate : 2.5sec
Waterproof : 30meter
With mercury switch (if bottom up, the light is turned up automatically)

5. FS-6

Size : 400mm x 71mm
Weight : 360g
Visibility : 6km
Battery Alkaline "D"cell : four
battery life : 40days
Flash rate : 2.5sec
Waterproof : 30meter

6. FS-7

LED lamps
Size : 385mm x 56mm
Weight : 330g
Visibility : 6km
Battery Alkaline "D"cell : four
battery life : 40days
Flash rate : 2.5sec
Waterproof : 30meter

SEA LIGHT

Fishing Net Indicating Lights
Photo sensitive strobe marker light

- Light sensitive switch : automatically activates as nighttime approach
- Multi-purpose buoy marker light(geo-seismic research, fishing,etc):also used to designate dangerous areas, construction and work areas, roadside emergencies or can be used as a general distress marker. 
- Lightweight, easy to install, simple to operate 
- Photocell activated: 'ON' at dusk, 'OFF' at dawn
- Material : Strong, impact resistant ABS resin 
- Color : International Orange, Yellow, Red 
- Beam Angle : Omni-directional 360' 
- Visibility : Hight intensity strobe is visible up to 3.7 miles(6km) 
- Lamp : Xenon flash tube

All Items have LED Lamp & Xenon Tube for Flash

1. F-1

LED lamps (LED red + white lamp)
Size : 128mm x 52mm
Weight : 96g
Visibility : 4km
Battery : Alkaline "D"cell : one
Battery life : 20days
Flash rate : 1.5sec
Waterproof : 20meter

2. F-2

not LED lamps
Size : 112mm x 52mm
Weight : 90g
Visibility : 4km
Battery : Alkaline "D"cell : one
Battery life : 20days
Flash rate : 1.5sec
Waterproof : 20meter

3. F-3

LED lamps (LED red + white lamp)
Size : 146mm x 41mm
Weight : 120g
Visibility : 4km
Battery : Alkaline "D"cell : one
Battery life : 20days
Flash rate : 1.5sec
Waterproof : 30meter

4. F-4

LED lamps (LED red + white lamp)
Size : 188mm x 52mm
Weight : 100g
Visibility : 4km
Battery : Alkaline "D"cell : two
Battery life : 20days
Flash rate : 1.5sec
Waterproof : 20meter

5. F-5

LED lamps (LED red + white lamp 22mm )
Size : 133mm x 51mm
Weight ; 100g 
Visibility : 4km
Battery Alkaline "D"cell : one
Battery life : 20days
Flash rate : 1.5sec
Waterproof : 10meter

6. F-6

not LED lamps, but 33mm lamp
Size : 133mm x 51mm
Weight ; 100g 
Visibility : 4km
Battery Alkaline "D"cell : one
Battery life : 20days
Flash rate : 1.5sec
Waterproof : 10meter

7. F-7

LED lamps (LED red + white lamp 22mm )
Size : 133mm x 51mm
Weight ; 100g 
Visibility : 4km
Battery Alkaline "D"cell : one
Battery life : 20days
Flash rate : 1.5sec
Waterproof : 10meter

8. F-8

not LED lamps
Size : 140mm x 39mm
Weight : 106g
Visibility : 4km
Battery : Alkaline "D"cell : one
Battery life : 20days
Flash rate : 1.5sec
Waterproof : 10meter

And other types are also available.

FISH LURING LIGHT

Fish Luring Lights are being used under the water to lure fishes.

Color : clear, luminous and other colors if requested

1. FL-1

Size : 159mm x 27mm
Weight : 45g 
Battery alkaline : AA 1.5V x 2pcs
Battery life : 3days
Flash rate : 1 sec
Waterproof : 50meter

We are supplying FL-1 with LED lamp(green, blue, red, yellow, etc) with snap which can go to 200meters under the water.

1. FL-2

Size : 111mm x 27mm
Weight : 30g 
Battery alkaline : AA 1.5V x 1pcs
Battery life : 1days
Flash rate : 0 sec
Waterproof : 50meter

FL-2 is not available with LED lamp

3. FL-3

Length: 185mm 
Weight: 125g 
Visibility: 1.5W 
Battery alkaline "C" 1.5V x 2pcs
Battery life: 30days 
Flash Rate : 0 sec 
Waterproof: 500meter

And other types are also available.