Main Line & Branch Line

Nylon Monofilament Line is being usually used at tuna long line system as main line or branch line.Our main lines and branch lines are all premium grade created by advanced high technology.

So our monofilament lines have good reputation for its exceptionally high strength, resistance, flexibility and clarity.

Our monofilament lines have full range of size available from 0.10mm to 4.50mm with superior tensile strength but they are still smooth and soft.And if requested,we can make any colors.
And because our monofilament lines have excellent flexibility and breaking strength, they can be used for a long time.

Kinds of Main Line and Branch Line

1. Nylon Monofilament Line

Main Line 
Branch Line : "SUPERUS"brand, "PRAEMIUM"brand

2. Fluoro Carbon Line

3. 1 x 8 Braided Tuna Line

4. Rope Main Line & Rope Branch Line

5. Nylon Braided Sekiyama Line

6. Wire Sekiyama Line ( for shark fishing )

Please your  inquiry  to Mrs. Schein : v-trading@farlep.net

NYLON MONOFILAMENT LINE.

MAIN LINE Color : natural, blue, pink and any colors if requested Being supplied in 5 nautical miles, or reels if regquested. Diameter(mm); Breaking Strength. 3.00; 900 lbs, 410 kgs. 3.10; 930 lbs, 420 kgs. 3.20; 960 lbs, 435 kgs. 3.30; 990 lbs, 450 kgs. 3.40; 1050 lbs, 475 kgs. 3.50; 1100 lbs, 500 kgs. 3.60; 1150 lbs, 520 kgs. 3.70; 1175 lbs, 532 kgs. 3.80; 1200 lbs, 545 kgs. 3.90; 1250 lbs, 612 kgs. 4.00; 1300 lbs, 590 kgs. 4.20; 1400 lbs, 636 kgs. 4.50; 1600 lbs, 725 kgs. BRANCH LINE We have been supplying two grade branch line, namely "SUPERUS" and "PRAEMIUM". Color : natural, green, pink, blue, gray, smoke, blood red, clear, black, etc. Being supplied in 500meter, 1,000 meter hank, or reels if requested. "PRAEMIUM" grade Diameter(mm); Breaking Strength. 1.20; 200 lbs, 90 kgs. 1.30; 230 lbs, 105 kgs. 1.40; 260 lbs, 120 kgs. 1.50; 270 lbs, 125 kgs. 1.60; 295 lbs, 130 kgs. 1.70; 310 lbs, 140 kgs. 1.80; 360 lbs, 165 kgs. 1.90; 390 lbs, 180 kgs. 2.00; 440 lbs, 200 kgs. 2.20; 500 lbs, 230 kgs. 2.50; 600 lbs, 270 kgs. "SUPERUS" grade Diameter(mm); Breaking Strength. 1.20; 250 lbs, 115 kgs. 1.30; 280 lbs, 125 kgs. 1.40; 300 lbs, 135 kgs. 1.50; 330 lbs, 150 kgs. 1.60; 375 lbs, 170 kgs. 1.70; 440 lbs, 200 kgs. 1.80; 485 lbs, 220 kgs. 1.90; 515 lbs, 235 kgs. 2.00; 560 lbs, 255 kgs. 2.20; 640 lbs, 290 kgs. 2.50; 835 lbs, 380 kgs.

# BIG GAME FLUORO CARBON LINE

Fluoro Carbon Line for Big Game with fast sinking characteristics, high shock resistance and incomparable impact strength can be most desirable alternative to Nylon Big Game Line to catch tuna, marlin and other saltwater giant fishes.

Fishermen can enjoy variety of this line sizes from 100lbs to 450lbs depending on target fishes.Soft and Transparent and Shock-free line

# PE ROPE

Color : white, red, green, brown, any colors if requested/Other size available

Typical Main Line

| Dia.(mm) | Length (M) | Weight(kg) | Breaking Strength (kgs) |
| --- | --- | --- | --- |
| 6.8 | 400 | 13.2 | 850 |
| 7.0 | 400 | 14.2 | 880 |
| 7.2 | 400 | 14.7 | 910 |

Typical Branch Line

| Dia. (mm) | Length (M) | Weight(kg) | Breaking Strength (kgs) |
| --- | --- | --- | --- |
| 4.0 | 300 | 4.1 | 220 |
| 4.2 | 300 | 4.3 | 280 |
| 4.5 | 300 | 4.7 | 310 |
| 4.8 | 300 | 5.2 | 330 |

8 - BRAIDED MAIN LINE

Well known and Typical Main Line

| Construction | Diameter(mm) | Length(M) | Weight(kg) | Breaking Strength(kg) |
| --- | --- | --- | --- | --- |
| 1.04mm x 8 | 4.2 | 1000 | 9.32 | 480 |
| 1.17mm x 8 | 4.5 | 900 | 10.37 | 560 |
| 1.28mm x 8 | 5.4 | 800 | 11.32 | 650 |

Diameter of Braided Main Line : from 4.2mm 

Clear and Any colors available

# NYLON BRAIDED SEKIYAMA LINE

With Lead core, or without Lead core

Special resin treatment

Color : red, blue, yellow, any colors if requested

The below spec is for Nylon braided Sekiyama Line with Lead Core

| Dia.(mm) | Length (M) | Weight(kg) | Breaking Strength (kgs) |
| --- | --- | --- | --- |
| 2.3 | 500 | 4.0 | 182 |
| 2.5 | 400 | 3.5 | 200 |
| 2.7 | 400 | 4.0 | 245 |
| 3.0 | 400 | 5.0 | 295 |
| 3.3 | 300 | 4.3 | 350 |
| 3.5 | 200 | 4.8 | 410 |
| 3.8 | 200 | 7.5 | 450 |
| 4.0 | 200 | 8.5 | 440 |
| 4.5 | 100 | 5.0 | 600 |